The SDV landscape is consolidating around centralized/zonal architectures, with Tesla maintaining a 3-5 year lead over legacy OEMs according to S&P Global Mobility. The critical differentiator is no longer hardware capability but software platform maturity and organizational agility-VW's Cariad has accumulated $7.5 billion in losses while Tesla iterates on FSD at fleet scale. For agentic AI-enabled SDV development, the winning pattern combines AUTOSAR Adaptive for safety-critical middleware, Eclipse SDV/COVESA for signal abstraction, and domain-specific AI agents orchestrated through standardized protocols like MCP or uProtocol.

This analysis examines 15+ platform providers and OEMs across four domains (AI, Cloud, Vehicle, Tool) to derive architectural recommendations for next-generation SDV development infrastructure.

# Part I: SDV Platform and Middleware Vendor Analysis

Sonatus dominates production deployments with 4M+ vehicles

Sonatus has achieved the largest production SDV deployment through its exclusive partnership with Hyundai Motor Group, with 4+ million vehicles across 24+ models including IONIQ6, EV9, and Genesis lineup. Their architecture centers on three differentiating capabilities that reduce integration complexity.

The SOA Adapter translates legacy CAN signals into discoverable SOME/IP, Shared Memory, and DDS services through cloud-configurable mapping-eliminating the need for firmware updates when signal definitions change. Their Network Shared Storage (NSS) provides centralized, cloud-managed in-vehicle data storage implementing data mesh patterns with policy-based collection. The Sonatus Command Center enables no-code automation deployment through Automator AI, allowing feature releases without new software builds.

Strengths: Production-proven at scale; comprehensive platform covering data collection to OTA; signal abstraction without firmware changes; TSN Profile 802. lDG compliance for zonal architectures.

Weaknesses: Heavy concentration on single customer (HMG); no explicit ISO 26262 product certification; proprietary approach versus open standards alignment.

## Sibros offers rapid deployment for OTA-focused architectures

Founded by ex-Tesla OTA platform engineers, Sibros provides the Deep Connected Platform achieving average deployment timelines under 6 months at 1/4 the cost of in-house builds. Their three-pillar architecture; Deep Logger, Deep Updater, Deep Commander-addresses the core connected vehicle requirements without full middleware ambitions.

The platform supports distributed, domain, and zonal E/E architectures through hardware-agnostic design validated on all major telematics control units. Notable deployments include Mahindra's Born Electric Vehicle platform and Bajaj's Chetak electric scooter fleet. Their SOVD standard compliance positions them well for next-generation diagnostics integration.

Strengths: Tesla engineering heritage; rapid deployment; architecture flexibility; strong compliance posture (ISO 27001, ASPICE).

Weaknesses: Narrower scope than competitors (OTA/connectivity focus); limited SOA middleware capabilities; smaller deployment scale.

## Apex.AI provides the only ASIL-D certified ROS 2-compatible middleware

Apex.AI has achieved what no other vendor has: ISO 26262 ASIL-D product certification (TUV Nord SEooC) for a ROS 2-compatible middleware stack. Their Apex.Grace (formerly Apex.OS) rewrites ROS 2 for deterministic, real-time execution with static memory pools eliminating runtime dynamic allocation.

Performance benchmarks demonstrate 4MB camera images in 48µs pub/sub on ARM Cortex-A53 under QNX OS for Safety. Their single API abstracts DDS, SOME/IP, MQTT, and CAN protocols, enabling hardware-agnostic development. Strategic investors include Continental (who co-develops ADAS/AD systems), ZF (5% stake), Toyota Ventures, and Volvo Group VC.

The Indy Autonomous Challenge victory (170+ mph autonomous racing) validated real-world performance, while integration with AUTOSAR Adaptive ara::corn provides enterprise adoption pathways.

Strengths: Highest safety certification in industry; ROS 2 ecosystem compatibility; open-source foundation (Eclipse iceoryx); multi-protocol abstraction.

Weaknesses: Requires underlying RTOS (QNX, INTEGRITY) for full certification; higher integration barrier for non-ROS organizations; smaller customer base versus established Tier 1s.

## RTI Connext DDS (Data Distribution Service) leads standards-based communication

RTI's Connext Drive powers 250+ autonomous vehicle programs with over 1 million vehicles running their software. Their December 2024 milestone-DDS adoption into all AUTOSAR packages and platforms-establishes DDS as a first-class citizen alongside SOME/IP in the automotive stack.

The XPENG deployment (all models from 2026, with P7 production since 2020) demonstrates Chinese OEM adoption of DDS for production autonomous driving. RTI's AUTOSAR Integration Toolkit provides zero-copy marshaling between AUTOSAR and DDS types with automatic ARXML-to-DDS-IDL code generation.

Strengths: Extensive certifications (TUV SUD ISO 26262 ASIL-D process, ISO 21434, ASPICE CLI); standards leadership; proven scale; COVESA VSS integration.

Weaknesses: Higher complexity than simpler messaging; commercial licensing model; focused on communication layer (not full middleware).

# Part II: OEM Software Platform Architectures

## Tesla's centralized architecture maintains industry-leading position

Tesla pioneered zonal architecture with Model 3, achieving significantly fewer ECUs than competitors and reducing wiring by 50%+. Their vertical integration strategy-custom FSD Computer chips designed in-house, manufactured by Samsung-enables rapid iteration impossible with supplier-dependent architectures.

The FSD V12 transition exemplifies their software advantage: 350,000+ lines of hand-coded rules replaced by a monolithic neural network processing images directly to steering/brakes/acceleration outputs. Fleet learning from 3+ billion FSD miles (January 2025) provides training data at a scale no competitor matches. Server-side remote configuration enables A/B testing across the entire fleet without OTA updates.

## Hardware Evolution:

| Generation | Process | Performance | Production |
| --- | --- | --- | --- |
| HW3 (FSD Chip) | 14nm Samsung | 72 TOPS | 2019+ |
| HW4 (FSD Computer 2) | 7nm Samsung | 3-8x HW3 | Jan 2023+ |
| AI5 | TBD | Next-gen | Expected 2026 |

Strengths: Industry-leading software iteration speed; lowest manufacturing complexity; continuous fleet learning; full vertical integration.

Weaknesses: HW3/HW4 version fragmentation; no formal L3 certification; camera-only approach controversy; Dojo supercomputer reportedly shut down (August 2025).

## BMW Neue Klasse launches most advanced legacy OEM architecture

BMW's Neue Klasse (series production 2025) implements four "superbrains" in a zonal architecture: Infotainment (BMW OS X), Automated Driving (20x computing power versus current generation), Driving Dynamics ("Heart of Joy"-100% in-house), and Core Functions (vehicle access, climate, lighting).

The architecture achieves 600 meters less wiring and 30% weight reduction through zonal organization with smart eFuses digitally replacing up to 150 traditional fuses. Their CodeCraft development environment supports 10,000+ developers producing 200,000 software builds per day at peak across 500+ million lines of code.

Critical differentiator: BMW developed the driving dynamics controller entirely in-house, maintaining their brand identity while outsourcing commodity functions. The same platform scales across BEV, PHEV, and ICE drivetrains.

Strengths: Fully in-house driving dynamics; technology scalability across powertrains; strong development infrastructure; clear execution.

Weaknesses: First production vehicle just launching; unproven at scale; high development investment required.

## Mercedes MB.OS achieves certified Level 3 autonomy

Mercedes-Benz is the only OEM with SAE Level 3 certified production vehicles (DRIVE PILOT), built on their four-domain MB.OS architecture: Infotainment, Automated Driving, Body & Comfort, and Driving & Charging. The chip-to-cloud architecture centers on NVIDIA DRIVE Orin (254 TOPS) with Luminar IRIS LiDAR for enhanced perception.

Their partnership strategy balances control and innovation: NVIDIA for compute and simulation (Omniverse digital twins), Luminar for next-gen LiDAR, Google for navigation, Qualcomm for infotainment, and Tencent for China cloud. Software-enabled revenue targets reach "high single-digit billion EUR" by end of decade.

Strengths: L3 autonomy leadership; balanced partnership strategy; premium positioning enables software revenue; production digital twin capabilities.

Weaknesses: External partnership dependencies; higher cost structure limits mass-market scaling; complex multi-partner integration.

## VW/Cariad pivots from in-house to partnership model

Cariad represents the automotive industry's most expensive software lesson: $7.5 billion operating losses (2022-2024) and E3 2.0 architecture cancelled in favor of a $5.8 billion Rivian joint venture. The E3 1.2 architecture now in production (Audi Q6 e-tron, Porsche Macan EV) was delayed approximately 2 years.

VW's new strategy segments the software stack into four areas: Driver Stack, Experience Stack, Cloud Stack, and Motion Stack, with external partnerships filling capability gaps. The Rivian N will deliver zonal architecture and software for first vehicles (VW ID.Every I) in late 2027, while Xpeng partnership addresses Chinese market requirements.

Key Lesson: Organizational complexity with multiple brand requirements (VW, Audi, Porsche, Skoda, Bentley, Lamborghini) proved incompatible with unified software platform development. 1,600 Cariad layoffs planned by end of 2025.

## Rivian's Gen 2 platform leads EV-native architecture

Rivian's Gen 2 platform (2024+) reduces ECUs from 17 to 7 powerful units organized into three zones (East, West, South), eliminating 1.6 miles of wiring per vehicle (44 lbs weight savings). Their born-digital advantage means no legacy systems to maintain-"Product today very different than 2 years ago" through continuous OTA evolution.

The $5.8 billion VW joint venture validates Rivian's architecture leadership while providing financial stability. First VW vehicles with Rivian software expected 2027, with R2 SUV production in 2026. Connected data platform built on AWS and Databricks enables fleet analytics at scale.

Strengths: Industry-leading zonal architecture; startup agility; VW backing provides stability; 1Ox more powerful AI prediction than previous generation.

Weaknesses: Production scaling challenges; not yet profitable; limited production history.

# Part III: Standards and Open-Source Frameworks

## AUTOSAR Adaptive Platform enables SDV patterns

AUTOSAR Adaptive (current release R25-11) provides the production-grade middleware foundation for SDV development on high-performance compute platforms. The POSIX-based architecture (PSE51 profile per IEEE 1003.13) with C++14 development supports multi-core SoCs running Linux or QNX.

## Core Functional Clusters:

- ara::com: Service-oriented communication with SOME/IP and DDS bindings

- ara::diag: Diagnostic Management implementing UDS and SOVD

- Update and Configuration Management: OTA-capable software deployment

- Cryptography: Security services with Adaptive Intrusion Detection

The key architectural shift from Classic Platform: runtime application installation/update enables OTA capabilities while dynamic service discovery allows flexible system composition. The December 2024 adoption of DDS into all AUTOSAR packages establishes multiple middleware options for different QoS requirements.

350+ partners including BMW, Bosch, Continental, Mercedes-Benz, Ford, GM, VW, and Toyota demonstrate industry-wide adoption. Hybrid architectures combining Classic (deeply embedded ECUs) and Adaptive (HPCs) are becoming standard.

## Eclipse SDV Working Group accelerates open-source adoption

The Eclipse SDV Working Group (founded 2022) has grown to 40+ member organizations with 20,457 commits, 388 contributors, 228 repositories in the past 12 months. Key members include Bosch, Microsoft, Continental, ZF, CARIAD, ETAS, NXP, and Toyota.

Eclipse Velocitas provides the vehicle app programming model with containerized deployment, SDK abstracting vehicle data access through generated Vehicle Models from VSS specification, and automated CI/CD workflows.

Eclipse Kuksa implements the vehicle abstraction layer as a central databroker with gRPC and VISS v2 protocol support, enabling hardware-agnostic signal access through the COVESA VSS data model.

Eclipse uProtocol bridges automotive and cloud communication domains through a transport-agnostic three-layer architecture supporting Eclipse Zenoh, SOME/IP, MQTT 5, and Android Binder transports. SDKs available for Rust, Java, C++, Python, and Kotlin.

## Integration Pattern:

[Vehicle App (Velocitas)] → [Databroker (Kuksa)] → [VSS Model] → [Signal Providers] → [CAN/Ethernet]

↓

[Service Registry (Chariott)] ← → [Other Services]

↓

[uProtocol Transport] → [Cloud/Mobile/Other ECUs]

## COVESA VSS provides industry-standard signal abstraction

The Vehicle Signal Specification (VSS) 5.0 (October 2024) defines a domain taxonomy for vehicle signals in hierarchical tree structure from root node through branches for Body, Cabin, Chassis, Powertrain, and ADAS.

CAN DBC to VSS Transformation Pattern:

- CAN DBC defines raw signal encoding (bit position, scaling, units)

- VSS provides semantic naming and standardized structure

- Signal Providers translate between CAN signals and VSS paths

- Databroker exposes VSS signals to applications

This abstraction enables applications coded against VSS to work across vehicles regardless of underlying CAN topology. AWS FleetWise, BlackBerry IVY, and Eclipse Kuksa all implement VSS, demonstrating cross-vendor adoption.

# Part IV: Cross-Domain Architectural Patterns from Robotics and AV

## NVIDIA platforms provide reference architectures for compute consolidation

NVIDIA DRIVE and Isaac platforms share architectural patterns directly applicable to SDV development. The Compute Graph Framework (CGF) manages data flow and execution scheduling across heterogeneous compute units (CPU, GPU, DLA, PVA), while the System Task Manager (STM) provides real-time scheduling with deterministic execution guarantees.

NVIDIA Thor (2,000 FP4 TFLOPS, production 2025+) exemplifies the centralized compute vision: multi-domain computing with hardware isolation enables simultaneous multi-OS execution (Linux, QNX, Android) while direct sensor fusion eliminates intermediary processors. The Transformer Engine delivers 1OX performance on ML models critical for perception and planning.

## Transferable Patterns

| Pattern | Source | SDV Application |
| --- | --- | --- |
| Layered Control Architecture | Boston Dynamics MPC | ECU consolidation, domain controllers |
| Hardware Abstraction Layer | DriveWorks SDK | Platform-agnostic software deployment |
| Compute Graph Framework | NVIDIACGF | Deterministic scheduling in zonal architecture |
| Fleet Leaming | Waymo SimulationCity | Connected vehicle data aggregation |
| Simulation-First Development | Isaac Lab | Virtual ECU testing, HIL replacement |

## Waymo demonstrates fleet learning at automotive scale

Waymo's simulation infrastructure validates simulation-first development: 20 million simulated miles per day (100+ years equivalent) derived from 20+ million real-world autonomous miles. Their Waymax open-source simulator (JAX-based) supports closed-loop testing with reactive agents-critical for edge case validation impossible in pure log replay.

The "Unified Driver" architecture deploys the same AI stack across cars (Waymo One) and trucks (Waymo Via), demonstrating platform reuse patterns applicable to SDV development. Fleet learnings propagate automatically through regular software updates.

## Real-time constraint handling patterns from robotics

Boston Dynamics' hybrid RL + MPC approach demonstrates multi-rate control applicable to vehicle systems:

- Strategic layer (1 Hz): Path planning and high-level decisions

- Tactical layer (10 Hz): Step patterns and trajectory adjustment

- Reactive layer (100+ Hz): Posture, balance, and actuator control

This layered architecture with temporal independence enables ISO 26262 compliance through clear WCET analysis and time-predictable software components. Freedom From Interference (FFI) via partitioning addresses mixed-criticality requirements central to zonal architectures.

# Part V: AI/ML Integration and Agentic AI Patterns

## Current Al/ML toolchain integration shows measurable productivity gains

McKinsey documents up to 40% time savings for writing, translating, and documenting code, with 44% productivity improvement for quality assurance tasks using AI assistance. A German tier-one supplier achieved 70% productivity gains using Gen AI for test vector generation (full branch coverage, MC/DC).

## Key Production Tools:

- GitHub Copilot+ CodeQL: CodeQL queries implementing CERT C++ and AUTOSAR C++ standards for ISO 26262 Part 6 compliance

- Vector VTT (Virtual Target): 54% time savings in build, download, and test stages versus traditional HIL; 99% reduction in testing stage time consumption

- aiMotive aiSim: First ISO 26262 ASIL-D certified automotive simulator with deterministic, physics-based simulation

- Applied Intuition HIL Sim: Large-scale HIL/SIL testing with CI/CD integration (Jenkins, Build.kite)

Chinese OEMs lead adoption: 65% of testing via software simulation versus 40-50% in other regions, with

75% of tests highly automated (versus 66% elsewhere).

## Agentic AI patterns emerge for automotive software development

McKinsey's State of AI 2025 reports 23% of organizations scaling agentic AI systems, with 39% experimenting. The HAL4SDV research project (European Chips Joint Undertaking funded) compared direct LLM prompting versus agentic approaches for querying large automotive software models, finding agent-based architectures achieve comparable accuracy with significantly more efficient token usage-critical for IP protection and regulatory compliance.

## Agent Orchestration Architectures:

- Vertical/Hierarchical: Conductor model (LLM-powered) oversees tasks and supervises simpler agents-ideal for sequential workflows but vulnerable to bottlenecks

- Horizontal/Decentralized: Agents work as equals in decentralized fashion-better for parallel processing but potentially slower

- Agentic AI Mesh (McKinsey recommendation): Interconnected agent networks with governance frameworks for autonomy levels and decision boundaries

## Interoperability Protocols:

  - Model Context Protocol (MCP) – Anthropic

  - Agent Protocol – LangChain

  - Agent2Agent – Google

  - Agent Network Protocol

## Fleet learning and privacy-preserving ML approaches

Federated Learning enables cross-fleet model improvement without raw data sharing:

  - Horizontal FL: Single-OEM and cross-OEM collaborative training

  - Vertical FL: Cross-industry collaboration

  - Vehicle OEM cloud serves as aggregation server

  - Only encrypted model updates (gradients) transmitted-GDPR compliant

Edge AI requirements (39% of stakeholders cite offline availability): Research advances "lightweight" models using pruning and simplification techniques, reducing model size by several orders of magnitude with minimal performance impact.

# Part VI: Four-Domain Architecture Framework Analysis

## AI Domain: Agent patterns and orchestration Level 1 Components:

  - LLM-based code generation agents (requirements -	code)

  - Test automation agents (scenario generation, coverage optimization)

  - Configuration management agents (signal mapping, variant validation)

  - Diagnostic agents (fleet-wide anomaly detection, predictive maintenance)

## Level 2 Patterns:

  - Agent orchestration via MCP/uProtocol for cross-domain coordination

  - RAG (Retrieval-Augmented Generation) for automotive knowledge bases (AUTOSAR specs, signal databases)

  - Multi-agent workflows for complex tasks (architecture design - implementation - testing)

  - Federated learning coordination for fleet model improvement

Recommended Architecture: Agentic AI mesh with domain-specific specialist agents coordinated through standardized protocols, governed by clear autonomy levels and human-in-the-loop checkpoints for safety-critical decisions.

## Cloud Domain: AWS-based vehicle backend services Level 1 Components:

  - MQTT broker infrastructure (Azure Event Grid, EMQX, AWS IoT Core)

  - Data lake architecture (Bronze/Silver/Gold processing tiers)

  - Fleet management services (OTA orchestration, diagnostic aggregation)

  - ML training infrastructure (federated learning coordination)

## Level 2 Patterns:

Vehicle MQTT Client → Event Grid MQTT Broker → Event Hub → Processing Functions → Data Lake

↓

Cosmos DB (State)

Reference Architecture (Microsoft Azure):

  - Claims Check pattern for large payloads (video, maps)

  - Store-and-Forward for intermittent connectivity

  - Tiered processing: High-priority (real-time) versus low-priority (analytics)

Scale Parameters (EMQ reference): IO-million level vehicle connections, million-level concurrent message throughput, sub-second latency for critical messages.

## Vehicle Domain: Embedded software patterns and real-time constraints Level 1 Components:

  - Central compute / HPC (NVIDIA Thor, Qualcomm Snapdragon Ride)

  - Zone controllers (4-8 per vehicle, I/0 consolidation)

  - Domain controllers (where domain architecture persists)

  - Smart sensors and actuators (edge processing)

## Level 2 Patterns:

  - E/E Architecture Selection:

| Architecture | ECU Count | Wiring | OTA | Development |
| --- | --- | --- | --- | --- |
| Distributed | 80-150 | Very High | Limited | Lower initial |
| Domain-Based | 30-50 | High | Moderate | Medium |
| Zonal | 10-20 | Medium | Good | Higher initial |
| Centralized | 3-5 | Low | Excellent | Highest initial |

  - Middleware Stack: AUTOSAR Adaptive for HPCs, Classic for real-time actuator control; SOME/IP primary with DDS for advanced QoS

  - Signal Abstraction: COVESA VSS + Kuksa Databroker pattern for hardware-agnostic applications

  - Diagnostics: UDS + SOVD coexistence; SOVD (RESTful, self-describing) for HPCs, UDS for legacy compatibility

## Tool Domain: Development toolchains and CI/CD Level 1 Components:

  - AUTOSAR configuration tools (Vector DaVinci, EB Tresos)

  - Simulation infrastructure (aiMotive aiSim, Applied Intuition HIL Sim)

  - CI/CD platforms (Jenkins, GitLab with automotive extensions)

  - Virtual ECU environments (Vector VTT, vECU generation)

## Level 2 Patterns:

  - CI/CT Pipeline (Vector reference):

Code Commit → Static Analysis (MISRA, AUTOSAR C++) → V-ECU Build → SIL Test → HIL Test → Report

  - Shift-Left Testing: Virtual target achieves 54% time savings versus physical HIL

  - AI-Augmented Tools: Copilot for code generation, AI-powered test selection/prioritization

  - Simulation Scale: Target 65%+ testing via simulation (Chinese OEM benchmark)

# Roadmap Recommendations for Agentic AI-Enabled SDV Development

## Near-term (12-24 months): Foundation building Priority 1: Establish signal abstraction layer

  - Implement COVESA VSS as canonical signal model

  - Deploy Eclipse Kuksa or equivalent databroker

  - Create CAN DBC  to VSS transformation pipeline

  - Enable hardware-agnostic application development

## Priority 2: AI-assisted development tools

  - Deploy GitHub Copilot with AUTOSAR C++/CERT C++ CodeQL rules

  - Implement AI-augmented test generation (30% productivity gain baseline)

  - Establish RAG system over AUTOSAR specifications and signal databases

  - Define governance framework for AI-generated code review

## Priority 3: Simulation infrastructure

  - Achieve 50%+ testing via simulation (path to 65% benchmark)

  - Implement Virtual ECU pipeline reducing HIL dependency

  - Establish deterministic scenario libraries (IM+ scenarios target)

  - Enable CI/CD integration with simulation infrastructure

## Medium-term (2-4 years): Platform maturation Priority 4: Agent orchestration architecture

  - Deploy domain-specific AI agents (test, configuration, diagnostic)

  - Implement agent communication via MCP or uProtocol

  - Establish multi-agent workflows for complex development tasks

  - Define autonomy levels and human-in-the-loop checkpoints

## Priority 5: Fleet learning infrastructure

  - Implement federated learning for cross-fleet model improvement

  - Deploy edge AI for in-vehicle inference (lightweight model patterns)

  - Establish data mesh governance for cross-domain analytics

  - Enable continuous model improvement from production fleet

## Priority 6: E/E architecture evolution

  - Target zonal/centralized architecture for new platforms

  - Implement AUTOSAR Adaptive on HPCs

  - Deploy SOME/IP+ DDS middleware stack

  - Achieve hardware/software lifecycle decoupling

Long-term (5+ years): Full autonomy

## Priority 7: Autonomous development workflows

  - AI agents handling end-to-end software lifecycle (requirements	+ deployment)

  - Cross-OEM collaboration via standardized federated learning protocols

  - Real-time adaptive systems with continuous fleet learning

  - Full simulation-first development with minimal physical prototypes

# Conclusion: Differentiation through platform velocity

The SDV competitive landscape reveals a clear pattern: organizational agility matters more than technical capability. Tesla's 3-5 year architecture lead stems not from superior hardware (competitors have access to equivalent silicon) but from vertically integrated software development velocity. VW's Cariad invested €27 billion yet fell years behind; Rivian with a fraction of the resources built industry-leading zonal architecture.

For agentic AI-enabled SDV development, three architectural decisions prove critical. First, adopt AUTOSAR Adaptive with COVESA VSS signal abstraction to enable hardware-agnostic development while maintaining safety certification pathways. Second, implement simulation-first development targeting 65%+ virtual testing to achieve the iteration speed Chinese OEMs demonstrate. Third, deploy domain-specific AI agents with clear governance frameworks rather than monolithic LLM approaches-the HAL4SDV research validates that agent architectures match accuracy while dramatically improving efficiency.

The four-domain framework (Al, Cloud, Vehicle, Tool) provides the organizational structure for this transformation. AI agents accelerate development; cloud infrastructure enables fleet learning; vehicle architecture supports OTA evolution; tool integration closes the feedback loop. OEMs that master this integrated approach-not just the technology, but the organizational operating model-will define the next generation of automotive software platforms.